| Ref | LSOG6 POP 120 Ref | Field Name | N/R/C/O | Negotiated Business Rules | Field Lengths | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N = Not Req’d R = Required C = Conditional O = Optional |  |  |  |  |
|  |  | Facility Availability Query (FAQ) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| FAQ1 | 1 | CCNA | R |  | 3 | a/n |  |
| FAQ2 | 6 | CC | O |  | 4 | a/n |  |
| FAQ3 | 2 | TXNUM | R | Co-Provider generated and may be reused one month after initial inquiry. | 22 | a/n |  |
| FAQ4 | 3 | D/TSENT XML: DT_TM_SENT | R |  | 12 | a/n | CCYYMMDDHHMinMin Military Time |
| FAQ5 | 4 | TXTYP | R |  | 1 | a | H = Facility Availability |
| FAQ6 | 5 | TXACT | R |  | 1 | a | A = New Inquiry |
| FAQ7 |  | SCATEG | R | Search Category: This field identifies which product the facility check is being done for. | 1 | a/n | P = POTS, PAL D = Design Service (DSO) H = Hi Capacity (DS1, DS3) I = ISDN U = Unbundled Loop |
| FAQ8 |  | QLR | R | Quantity of Lines requested: This field identifies the number of lines needed. QLR must be greater than zero when SCATEG = P, D, H, or I. QLR must be zero when SCATEG = U. | 2 | n | 1 - 99: 1 when checking facilities for Point A and/or Z on point to point Resale Private Line. 2-99: if checking facilities for multiple circuits at same address for Multi-point Resale Private Line. |
|  |  | Resale Private Line or Frame Relay Section. This section applies if Service Category is H or D. |  |  |  |  |  |
| FAQ9 | 60 | CS | C | Required if SCATEG = H or D. | 5 | a/n | Applicable USOC. |
| FAQ10 |  | SERCD | C | Service Channel Code: This field identifies the first two characters of the NC that is related to Hi Capacity or Design Service facilities. Required if SCATEG = H or D. | 2 | a | First two digits of NC. |
| FAQ11 |  | USOC | C | Universal Service Order Code: This field identifies a code that is associated with a change for the service. Required if SCATEG = H or D. | 5 | a/n |  |
| FAQ12 |  | MUXLOC | O | MUX Location: This field identifies the CLLI Code for the wire center. Optional if SCATEG = H or D. Co-Provider must determine if MUXing is necessary for the desired CLSERV, SERCD, and USOC. | 8 | a/n | CLLI code for wire center location. |
|  |  | Telephone Number Address Circuit Indicator Section This applies for all Service Categories (SCATEG). |  |  |  |  |  |
| FAQ13 |  | TNADDRCKTIND | R | Telephone Number Address Circuit Indicator: This field states if a TN, Address or Circuit is being used to do the search. If SCATEG = I, H or D, then TNADDRCKTIND must be A. If SCATEG = U, then TNADDRCKTIND may be A, T, or C. If SCATEG = P, then TNADDRCKTIND must be A or T. | 1 | a/n | T = TN A = Address C = Circuit ID |
| FAQ14 | 54 | WTN | C | Required if TNADDRCKTIND = T; must be existing TN at the end user address. | 12 | a/n | Dashes in positions 4 and 8. |
| FAQ14a |  | NUMPT |  | Number of Ports: This field identifies if the facility check is for one or two locations. Populated by CenturyLink. | 2 | n | 1: Facility Avail for one location, (Location A). 2: Facility Avail for two locations, (Location A and Z) |
|  |  | Fields AFT through CALA repeat as a group for SCATEG = H or D. Fields AFT through LSO are a group if SCATEG = P, U or I. |  | If SCATEG = H or D, then this section is required and may repeat up to 2 times, If SCATEG = P, U, or I, then this section is required and cannot repeat. |  |  |  |
| FAQ15 | 14 | AFT* | O |  | 1 | a/n | A = Rural Route or Route and Box number B = Unnumbered C = Provider assigned house number D = Descriptive |
| FAQ16 | 15 | SAPR* | O |  | 5 | a/n |  |
| FAQ17 | 16 | SANO* | C | This field is used for Numbered address search. Co-Provider must populate this field with the house number of the service address for the numbered address search (e.g. 123). This field along with the SASF, SASN, SASD and SATH fields, gives the complete street address (e.g. 123-1A North Maine Street). This field is required when SAPR or SASF are populated. The AHN, ROUTE or BOX Fields can not be populated. | 8 | a/n |  |
| FAQ18 | 18 | SASF* | O | Optional for TNADDRCKTIND = A and address is numbered. Address must have already been validated and must be exact match. | 4 | a/n |  |
| FAQ19 | 19 | SASD* | O |  | 2 | a | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| FAQ20 | 20 | SASN* | C | Required if TNADDRCKTIND = A. Address must have already been validated and must be exact match. Use field to determine type of address search: Numbered: Does not begin with ", " and is populated with a street name. Example: Broadway. SANO must be populated and SASF may be populated. AHN, ROUTE, or ROUTE and BOX field cannot be populated. Unnumbered: Begins with ", " followed by a street name or the community name, or with ", , " followed by the community name. Example: , Vail. In addition, one of the following fields is required: AHN, ROUTE, or ROUTE and BOX. Descriptive: Begins with ", " followed by a description of the address. Example: , Mission Apts. The following fields cannot be populated: LD1, LV1, LD2, LV2, LD3, LV3, AHN, ROUTE, or ROUTE and BOX. The CITY, STATE and ZIP or CALA must be populated for all of these three address searches. This field is required when the SASD, SATH, or SASS fields are populated. | 50 | a/n |  |
| FAQ21 | 21 | SATH* | O |  | 10 | a/n |  |
| FAQ22 | 22 | SASS* | O |  | 4 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| FAQ23 | 23 | LD1* | C | Required when LV1 is populated, otherwise prohibited. | 4 | a/n | APT LOT RM SLIP UNIT SUIT |
| FAQ24 | 24 | LV1* | C | Required when LD1 is populated, otherwise prohibited. | 10 | a/n |  |
| FAQ25 | 25 | LD2* | C | Required when LV2 is populated, otherwise prohibited. | 4 | a/n | FLR |
| FAQ26 | 26 | LV2* | C | Required when LD2 is populated, otherwise prohibited. | 10 | a/n |  |
| FAQ27 | 27 | LD3* | C | Required when LV3 is populated, otherwise prohibited. | 4 | a/n | BLDG WNG PIER |
| FAQ28 | 28 | LV3* | C | Required when LD3 is populated, otherwise prohibited. | 10 | a/n |  |
| FAQ29 |  | AHN* | C | Assigned House Number: This field identifies an address without a valid street number. Can only be populated if SASN begins with an ', ' sign. The SANO Field can not be populated. | 8 | a/n |  |
| FAQ30 |  | ROUTE* | O | Rural Route: This field identifies the service address route. Not allowed if SCATEG = I or U. The SANO Field can not be populated. | 3 | a/n |  |
| FAQ31 |  | BOX* | O | Box Number: This field identifies the service address box. Not allowed if SCATEG = I or U. The SANO Field can not be populated. | 12 | a/n |  |
| FAQ32 | 29 | AAI* | N | Not used by CenturyLink. | 100 | a/n |  |
| FAQ33 | 30 | CITY* | C | Required if TNADDRCKTIND = A. Address must have already been validated and must be exact match. | 32 | a/n |  |
| FAQ34 | 31 | STATE* | C | Required if TNADDRCKTIND = A. Address must have already been validated and must be exact match. | 2 | a/n |  |
| FAQ35 | 32 | ZIP* | C | Required when TNADDRCKTIND = A, T or C and CALA is not populated. | 5 | n |  |
| FAQ36 |  | CALA* | C | Customer Address Location Area: Code used to identify what area an address is located in. Required when TNADDRCKTIND = A, T or C and ZIP is not populated, or if the ZIP is populated but crosses multiple CALAs. | 4 | a | BLVU CHY CO CPR DNV EAST EUGN IA ID IDO MAL MN MT MTA NCO ND NE NMX OLYM OREA PNX PORT SCO SD SEAT SPOK TSN UT UTA VANC WI WO WY WYO |
| FAQ37 | 45 | LSO* | C | Required if SCATEG = U and TNADDRCKTIND = C. Required if SCATEG = P or I. Not required if SCATEG = D or H. | 6 | n | Leading NPA NXX of Central Office; returned in address validation response NPA/NXX of the switch |
| FAQ38 |  | LOOPCHAR | C | Loop Characteristics: This field identifies the type of loop the facility check is being done for. Required if SCATEG = U. | 1 | a | L = Loaded N = Non-loaded D = Does not matter |
| FAQ39 | 63 | ECCKT | C | Required if SCATEG = U and TNADDRCKTIND = C, otherwise not applicable. | 42 | a/n |  |
|  |  | CFA Section This Section is optional if SCATEG= H or D. |  |  |  |  |  |
| FAQ40 |  | FACDES | O | Facility Designation: This filed identifies the designation of the CFA. | 5 | a/n |  |
| FAQ41 |  | FACTYP | C | Facility Type: This field identifies the facility type related to the CFA. Required if FACDES is present. | 6 | a/n |  |
| FAQ42 |  | CHAN | C | Channel: This field identifies the type of channel related to the CFA. Required if FACDES is present. | 2 | n | DS1 = 1-24 DS3 = 1-28 |
| FAQ43 |  | LOCA | C | Location A: This is the originating CLLI Code. Required if FACDES is present. | 11 | a/n | CLLI code applying to the service location. |
| FAQ44 |  | LOCZ | C | Location Z: This is the terminating CLLI Code. Required if FACDES is present. | 11 | a/n | CLLI code applying to the service location. |
|  |  | Facility Availability Response (FAR) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| FAR1 | 1 | CCNA | R |  | 3 | a/n | Echo from query |
| FAR2 | 6 | CC | O |  | 4 | a/n | Echo form query |
| FAR3 | 2 | TXNUM | R |  | 22 | a/n | Echo from query |
| FAR4 | 3 | D/TSENT XML: DT_TM_SENT | R |  | 12 | a/n | CCYYMMDDHHMinMin Military Time |
| FAR5 | 4 | TXTYP | R |  | 1 | a | H = Facility Availability |
| FAR6 | 5 | TXACT | R |  | 1 | a | A = New Inquiry |
| FAR7 |  | RESPONSE | R | Response: This field identifies if ‘Good’ or ‘Bad’ response is being returned. | 1 | a/n | G = Good B = Bad |
| FAR8 |  | SCATEG | R | Search Category: This field identifies which product the facility check is being done for. | 1 | a/n | Echo from query |
| FAR9 |  | QLR | R | Quantity of Lines Requested: This field identifies the number of lines needed. | 2 | n | Echo from query |
|  |  | POTS Section Information present only if RESPONSE = G and SCATEG = P. |  |  |  |  |  |
| FAR10 |  | WLINUM | C | Working Lines Left In: This field identifies the number of lines left on the facility. Required if SCATEG =P. | 2 | n | 0 - 99 |
| FAR11 |  | DSVC | R | Dual Service Availability: This field identifies the Dual Service Availability | 1 | a/n | 0 = Not Available 1 = Available |
| FAR12 |  | REMINDER | C | Dual Service Reminder: This field is required if DSVC=1 otherwise prohibited. | 38 | a/n | REMINDER: Check both service addresses |
| FAR13 |  | LINESTAT* | C | Line Status: This field identifies the state of the lines requested. If SCATEG=P, the fields LINESTAT to REMARK may repeat QLR times. | 1 | a/n | A = Available H = Held Order S = Available (SDT) |
| FAR14 | 36 | DSIND* | C | If SCATEG=P, this field repeats QLR times. | 1 | a/n | Y = Yes N = No |
| FAR15 |  | REMARK* | C | Remark: This field identifies additional information related to the lines requested. If SCATEG=P, this field repeats QLR times. | 255 | a/n |  |
| FAR16 |  | PRDNOTSUPP* | C | Products Not Supported: This field identifies what is not supported within the central office switch. Returned only if the central office switch does not support certain products/features. | 100 | a/n | Message Waiting Indicator - Visual not supported Dual Service not supported Caller ID not supported |
|  |  | POTS Pending Order Section Returned only if an order is pending for that end user address. Present only if RESPONSE = G and SCATEG = P. |  |  |  |  |  |
| FAR17 |  | PDORDERTYP* | O | Pending Order Type: This field identifies the type of order that is pending at the end user address. | 2 | a | C = Change N = New T = Outside Move D = Disconnect F = From R = Record Changes |
| FAR18 |  | ORDNUM* | O | Order Number: This field identifies the order number related to the pending order type. | 8 | a/n |  |
| FAR19 |  | PDORDERDD* | O | Pending Order Due Date: This is the due date for the pending order. | 8 | a/n | Format: CCYYMMDD |
|  |  | Private line or FRAME RELAY Section This section is present only if RESPONSE = G and SCATEG = D or H. |  |  |  |  |  |
| FAR20 |  | LOCANFACS | C | Location A Facility Status: Required if Location A address is present on query and CFA Section is not present. | 2 | n |  |
| FAR21 |  | LOCARMKS | C | Location A Remarks: Required if Location A address is present on query and CFA Section is not present. | 255 | a/n |  |
| FAR22 |  | LOCZNFACS | C | Location Z Facility Status: Required if Location Z address is present on query. | 2 | n |  |
| FAR23 |  | LOCZRMKS | C | Location Z Remarks: Required if Location Z address is present on query. | 255 | a/n |  |
| FAR24 |  | MUXST | C | MUX Status: Required if MUXLOC is present on query. | 1 | a | Y = MUX Available N = MUX Not Available BLANK = MUX not Queried |
| FAR25 |  | CFAST | C | CFA Status: Required if CFA Section present on query. | 12 | a | CFA OK or CFA CONFLICT |
|  |  | ISDN Loop Qualification Section This section is present only if RESPONSE = G and SCATEG = I. |  |  |  |  |  |
| FAR26 |  | ISDNFLAG | C | Integrated Switched Digital Network (ISDN) Indicator: This field identifies if facilities can support ISDN. Required if SCATEG=I. | 1 | a/n | Y = ISDN Capable N = Not ISDN Capable |
| FAR27 |  | SN56FLAG | C | Switched Net 56 Indicator: This field identifies if facilities are Switched Net 56 Capable or not. Required if SCATEG=I. | 1 | a/n | Y = Switched Net 56 Capable N = Not Switched Net 56 Capable |
| FAR28 |  | CC64FLAG | C | 64K Clear Channel Indicator: This field identifies if facilities are 64kbps Clear Capable or not. Required if SCATEG=I. | 1 | a/n | Y = 64kbps Clear Channel Capable N = Not 64kbps Clear Channel Capable |
| FAR29 |  | X75FLAG | C | X75 Packet Service Indicator: This field identifies if the facilities are X.75 Capable or not. Required if SCATEG=I. | 1 | A/n | Y = X.75 Capable N = Not X.75 Capable |
| FAR30 |  | LINESNUM* | C | Number of Lines Required if SCATEG = I. | 2 | n |  |
| FAR31 |  | DESC* | C | Description Required if SCATEG = I. | 500 | a/n |  |
|  |  | Unbundled Loop Section This section filled in only if RESPONSE = G and SCATEG = U. |  |  |  |  |  |
| FAR32 | 63 | ECCKT* | O |  | 42 | a/n |  |
| FAR33 |  | SERVON* | O | Served On: The type of loop (BRI, Analog, etc.) If SERVON* is FTTP you cannot resell data services. | 20 | a/n | COPPER FIBER FTTP |
| FAR34 |  | LOAD* | O | Loading: The capacity of the loop. | 15 | a/n |  |
| FAR35 |  | MOVE* | O | Can be moved to: The indication whether POTS can actually be moved to Unbundled Loop and whether loading/unloading is required. | 60 | a/n |  |
|  |  | Error Section This section filled in only if RESPONSE = B. |  |  |  |  |  |
| FAR36 |  | ERRCODE* | C | Error Code: Indicates a predetermined error code. | 14 | a/n | Format (Character positions): 1-3 : Form Name 4-11 : Field Name 12-14 : Error Code |
| FAR37 |  | ERRMESG* | C | Error Message: Indicates additional information about the error. | 255 | a/n |  |